# LABSHEET 1: INTRODUCTION TO 8085 MICROPROCESSORS

Name: Vinayak V Thayil                                              Roll Number: AM.EN.U4CSE21161

- Convert the binary number 01000101 to hexadecimal.

- Convert the hex number 0x63F to binary.

- Give the signed representations of the decimal 45 in 8-bit binary and hexadecimal.

- Specify the signed and unsigned decimal representations of the 8-bit hex number 0x95.

- Convert the signed binary number (11011010)2 to signed decimal.

- For the unsigned 8-bit number system, the basis elements are {128,64,32,16,8,4,2,1}. What are the basis elements of signed 8-bit number system?

- Give the representations of -54 in 8-bit binary and hexadecimal.

- What are the possible values of 8-bit signed numbers?

- What are the possible values of 8-bit unsigned numbers?\

- Download a simulator for simple 8-bit processor 8085 in the below link https://gnusim8085.github.io/

- Fill the machine code for the following assembly program

| Assembly | Instruction size | Memory Address | Object Binary Code | Object Code in Hex |
| --- | --- | --- | --- | --- |
| Code1: MVI A, 32H MVI B, 48H ADD B OUT 01H HLT | 2 bytes 2 bytes 1 byte 2 bytes 1 bytes | 4200 4202 4204 4205 4207 | 0011 1110 0011 0010 0000 0110 0100 1000 1000 0000 1101 0011 0000 0001 0111 0110 | 3E 32 06 48 80 D3 01 76 |
| Code2: MVI A,01H STA 4500H HLT | 2 bytes 3 bytes 1 byte | 4200 4202 4205 | 0110 1110 0000 0001 0011 0010 0000 0000 0100 0101 0111 0110 | 3E 01 32 00 045 76 |

| Code 3: LDA 1000H MOV B, A LDA 2000H STA 1000H MOV A, B STA 2000H HLT | 3bytes 1 byte 3 bytes 3 bytes 1 byte 3 bytes 1 byte | 4200 4203 4204 4207 420A 420B 420E | 0011 1010 0000 0000 0001 0000 0100 0111 0011 1010 0000 0000 0010 0000 0011 0010 0000 0000 0001 0000 0111 1000 0011 0010 0000 0000 0010 0000 0111 0110 | 3A 00 010 47 3A 00 020 32 00 010 78 32 00 020 76 |
| --- | --- | --- | --- | --- |
| Code 4: MVI A,55H CMA STA 1001H MVI A,00H HLT | 2 bytes 1 byte 3 bytes 2 bytes 1 byte | 4200 4202 4203 4206 4208 | 0011 1110 0101 0101 0010 1111 0011 0010 0000 0001 0001 0000 0011 1110 0000 0000 0111 0110 | 3E 55 2F 32 01 010 3E 00 76 |